# Calibration Tool Manual

- Commands:

| Command<Uncased> | Functions | Details |
| --- | --- | --- |
| Help | Show the usage |  |
| CAGM | Calibrate Acc & Gyro & Mag Data | Sample the raw data of the three IMU sensor devices, then calculate and generate a calibration data : “calibration_agm.bin” |
| CALS | Calibrate Ambient Light Sensor Data | Sample the data from Sensor API, then calculate and generate a calibration data : “calibration_als.bin” |
| CALL | Calibrate AGM and ALS | Generate a calibration data : “calibration_all.bin” |
| FAGM | Flash ACC, GYRO, and MAG calibration data into BIOS | Flash the data of AGM in “calibration_agm.bin” into Firmware/BIOS |
| FALS | Flash ALS calibration data into BIOS | Flash the data of ALS in “calibration_als.bin” into Firmware/BIOS |
| FALL | Flash AGM and ALS calibration data into BIOS | Flash the data of AGM and ALS in “calibration_all.bin” into Firmware/BIOS |

Note:

- Do the calibrations before you install new system!

- Flash the calibration data into firmware to make it work!

- Run as Administrator!

- Reboot OS to make the calibration data take effect!

- CAGM command

The detail steps of calibrating AGM :

Step 1 / 5:

Keep the device horizontal with the screen up, and make the Axis-Y pointing to Magnetic North.

Step 2/ 5:

Keep the device horizontal with the screen up, and make the Axis-Y pointing to Magnetic South.

Step 3 / 5:

Keep the device horizontal with the screen down, and make the Axis-Y pointing to Magnetic North.

Step 4 / 5:

Lay the device horizontal, then rotate the device CLOCKWISE.

Step 5 / 5:

Lay the device Vertical with the windows button at bottom, then rotate device CLOCKWISE.